# Manatee County Government Development Services Department

This document was developed to provide educational information to the general public. It will assist with some of the common questions asked on a daily basis. The information below is for educational purposes only and does not include all scopes of work that requires/does not require a permit. It is also not limited to or incorporate all requirements or details. For further explanation and additional information contact the Development Services Department, at 941-748-4501 Ext. 3800 or visit our website at www.mymanatee.org. Development Services is open Monday – Friday 8:00am till 4:00pm.

### The following activities require a State Certified or Locally Licensed Contractor and a Building Permit:

(An owner can serve as a contractor providing the building or residence is for their own use or occupancy. It may not be built or substantially improved for sale or lease. If a building or residence that has been built or substantially improved by an owner is sold or leased within 1 year after the construction is complete, the law will presume that the owner built or substantially improved it for sale or lease, which violates this exemption.) FS 489.103

##### Building:

- New Roofs and re-roofs (mobile homes themselves not covered under FBC- license required but no permit) *see item 18 on next page

- Windows & Doors (Includes new openings and closing of existing opening)

- Attached Decks 120 sq.ft. or larger and 30” or higher (with or without a roof)

- Ramps and elevated walkways

- New/Replacement of siding.

- Kitchen remodel (only if adding or altering existing electrical, plumbing, building, or walls)

- Renovation of existing structures

- Masonry fences/walls and columns (Residential and Commercial)

- Drywall (work below base flood elevation plus one foot, firewalls, load bearing walls, and involves work to plumbing, electrical, or air conditioning)

- Replacement/Repair of rotten wood (only if load bearing or structurally significant)

- New/Replacement of fascia and soffits.

- Ceiling fans (If new wires are installed or wires are connected to existing wires)

- Additions (including mobile homes)

- New carport, porch, and screen room (No permit required on screening of existing porch or lanai with existing roof and floor/slab). If any portion of the walls are closed in with panels, etc. or kick plate is higher than 12 inches will require a permit.

- Sheds (includes prefabricated) * see activities that do not require a permit below on item 17*

- Barn/Animal shelters (An exemption permit required if it is a bona fide working farm and having a FEIN)

- Concrete slab with footers

- Swimming pools (including above ground pools that exceed 24” inches in height)

- Foam Insulation on Residential structures

- Foam Insulation on Commercial structures

- Demolition of structures/pools

- Paint Booths

- Any detached deck over 30 inches and/or over 120 square feet requires a permit. All attached decks require a permit. Detached Decks that are under 30 inches in height and under 120 square foot in size.

##### Electrical:

- Any alteration of electrical

- Residential low voltage (wired or wireless) (No permit required unless cost exceeds $5000 within any 12-month period)

- T-Pole on Construction Trailers

- T-Pole for Irrigation and Wells

- Service Changes (Residential/Commercial)

##### Mechanical:

- Replacement and New Air Conditioners

- Replacement and New Install Duct Work

- Range Hoods

- Walk-In Coolers

- Replacement and New Exhaust Fans

##### Gas:

- Natural Gas Line install

- Propane Tank install

- Natural Gas/Propane Gas Water Heater Install

- Natural Gas/Propane Gas Pool Heater Install

##### Plumbing:

- Tankless water heater replacement/new install

- Water heater replacement/new install *see activities that do not require a permit #2 below

- Backflow replacement/new install

- Re-pipe

- New water service

- New sewer/replacement

#### The following activities do not require a permit:

##### Building:

- Painting

- Wallpaper and other wall coverings

- Rescreening, any repairs of hardware or fasteners but requires a license.

- Pressure cleaning

- Trim carpentry

- Flooring (tile, carpet, laminate, wood, etc.)

- Paneling (Indoor nonstructural)

- Shelving (New Commercial may require a permit)

- Popcorn ceilings (remove and refinish)

- Cabinet installation

- Ceiling fans (if plugged into an existing electrical outlet)

- Residential home alarms (wired or wireless – if under $5000)

- Replacement of old drywall to new (Installation or replacement of drywall if the contract does not include work on load bearing walls, structures within flood hazard areas or any plumbing, electrical, or air conditioning work.)

- Non foam Insulation on residential.

- Non-Structural residential fences, such as wood, chain link and vinyl are exempt from obtaining a building permit when located on a one or two-family lot that is not located in a flood hazard area.

- Window in Mobile homes (Permit is required for any additions to the original Mobile Home)

- Prefabricated sheds used as tool and storage, playhouses and similar uses provided the floor area does not exceed 120 square feet and not over 8 feet high, calculated from mean roof height, however zoning and flood regulations will still apply.

- Roof Overs (not free standing), re roofs on mobile homes as delivered from factory do not need a permit but does require a license. Any addition to the Mobile home does require permit and license.

- Installation or repair od satellite antennas that are one meter (approx. 39 inches) or less in diameter which are located on commercial or industrial properties. For residential properties, the installation or repair of satellite antennas attached to primary structures which are one meter (approx. 39 inches) or less in diameter are exempt from permit requirements. Note: permit exemption does not exclude dishes from being safely installed.

- Playground equipment (swings, slides, monkey bars, basketball hoops, doghouses and pump covers) for residential use. All equipment and structures must meet zoning setbacks. Any electric service to such playground equipment or any structural element such as footers/structural slabs requires a permit and compliance with the Florida Building Code.

- Detached Decks that are under 30 inches in height and under 120 square foot in size. Any detached deck over 30 inches and/or over 120 square feet requires a permit. All attached decks require a permit.

- Non-structural concrete/paver patio

##### Electrical:

- Repair or replacement of electrical wall switches and wall outlet devices (receptacles) 20 amps and 120 volts or less.

- Low-voltage electrical systems in single-family residences, duplexes and townhouses (excluding condos)

- Portable heating appliances

##### Mechanical:

- Replacement of window air conditioning unit

- Installation of ductless range hoods

- Portable heating appliances

- Portable cooling unit

- Steam, hot or chilled water piping within any heating or cooling equipment regulated by this code

- Replacement of any part which does not alter its approval or make it unsafe

- Portable evaporative cooler

- Self-contained refrigeration system containing 10 pounds (4.54 kg) or less of refrigerant and actuated by motors of one-horsepower (746 W) or less

- The installation, replacement, removal or metering or any load management control device

- Repair of residential air conditioning duct

- Repair/replacement of residential coil, compressor or refrigerant piping by licensed air conditioning or mechanical contractor

- Repair/replacement of residential non-combustion heating by licensed air conditioning or mechanical contractor

- Repair of residential ventilation fans

##### Gas:

- Portable heating device

- Replacement of any minor part that does not alter approval of equipment or make such equipment unsafe

##### Plumbing:

- Replacement of plumbing fixtures such as toilets and faucets providing no rough plumbing has been changed

- Water heaters change out - electric to electric ONLY, same size, same location (commercial and residential)

- Irrigation only (Backflow replacement - See requirements for back-flow requirements for license and permits)

- Stopping of leaks in drains, water, soil, waste or vent pipes provided, however, that is any concealed trap drain pipe, water, soil, waste or vent pipe becomes defective and it becomes necessary to remove and replace the same with new material, such work shall be considered new work and a permit shall be obtained and inspection made as provided in this code.

- The clearing of stoppages or the repairing of leaks in pipes, valves or fixtures, and the removal and reinstallation if water closets, provided such repairs do not involve or require the replacement or rearrangement of valves, pipes or fixtures.

- Repair/replacement of residential dishwasher

- Repair/replacement of residential disposal

- Repair of residential irrigation system

- Repair of residential domestic water service/lines

- Repair to building sewer line (excluding repair or replacement of interceptors/separators, by licensed plumbing contractors)

- Repair/replacement of residential pool equipment other than pool heater

- Repair of residential shut-off valve on a domestic water line

# HB 803 (Effective July 1, 2026)

Residential Construction Permit Exemptions

Under HB 803, certain non-structural residential construction projects with a total value of less than $7,500 may qualify for a permit exemption. However, the exemption does not apply to:

  - Electrical work

  - Plumbing work

  - Mechanical work

  - Gas work

  - Structural work including (Railings, guards, pool barrier/fences, windows and doors, siding and soffits, roofing, elevated decks are all items with structural requirements and considered structural)

  - Any work on a property located partially or entirely within a flood hazard area as defined by the Florida Building Code

## State law also prohibits dividing a larger project into multiple smaller projects to avoid permitting requirements.

# To qualify for the exemption under subparagraph 1., the owner or the owner's contractor must submit a written request for exemption to the local enforcement agency with a copy of the contract or other documentation demonstrating the nature of the work to be performed.

# Temporary Flood Barrier Exemptions

HB 803 also establishes a permit exemption for certain temporary flood walls and barriers installed at single-family homes, two-family dwellings, and townhouses. To qualify, following conditions must be met:

- The wall or barrier is non-load-bearing and non-habitable.

- The wall or barrier is installed on the residential property of a single-family dwelling, two-family dwelling, or townhouse.

- The wall or barrier is constructed to mitigate or prevent storm surge or floodwater from entering a structure or property.

- The wall or barrier is installed by a contractor licensed under Part I of Chapter 489, Florida Statutes.

## The wall or barrier complies with all applicable local zoning, drainage, easement, and setback requirements.

Properties located partially or entirely within a flood hazard area are not eligible for this exemption.

## To qualify for the exemption under subparagraph 1., the owner or the owner's contractor must submit a written request for exemption to the local enforcement agency with a copy of the contract or other documentation demonstrating the nature of the work to be performed

## Documentation Required On-Site

For all approved exemptions, a copy of the exemption request stamped and approved by the Building Official must remain on-site throughout construction and be available for review by Code Enforcement personnel. Failure to maintain the approved exemption documentation on-site may result in the issuance of a Stop Work Order.

## Important Notice Work performed under an approved permit exemption will not be inspected by County staff. Property owners and contractors are responsible for ensuring all exempt work complies with applicable laws, regulations, and building standards.

#### Types of Contractor Licenses offered by Division of Business and Professional Regulations (DBPR)

(Contractors shall establish a Qualification File with the County prior to performing any work). Please note: Manatee County does not Reciprocate Registered licenses. All Registered license holders must be current and active in Manatee County. To verify a license please contact the Licensing Department – licensing@mymanatee.org or 941-748-4501 Ext 3574.

- General (CG & RG)

- Building (CB & RB)

- Residential (CR & RR)

- Sheet Metal (CS & RS)

- Roofing (CC & RC)

- Air Conditioning “Class A” (CA & RA)

- Air Conditioning “Class B” (CA & RA)

- Mechanical (CM & RM)

- Commercial Swimming Pools/Spa (CP& RP)

- Residential Swimming Pool/Spa (CP & RP)

- Swimming Pool/Spa Servicing (CP & RP)

- Plumbing (CF & RF) Precision Tank Tester

- Underground Utility and Excavation (CU)

- Solar (CV)

- Pollutant Storage System (PC)

- Specialty (SCC, RX)

  - Gypsum Drywall

  - Demolition (Voluntary/Limitations) *

  - Glass and Glazing (Voluntary) *

  - Irrigation (Voluntary)

  - Industrial Facilities

  - Pollutant Storage System

  - Tank Lining Applicator

  - Structural Aluminum or Screen Enclosures

  - Structural Masonry

  - Structural pre-stressed, precast concrete

  - Gas Line

  - Marine

  - Marine Seawall

  - Marine Bulkhead

  - Marine Dock

  - Marine Pile Driving

  - Precision Tank Tester

  - Residential Pool/Spa Servicing

  - Solar Water Heater

  - Structure

  - Swimming Pool Decking

  - Swimming Pool Excavation

  - Swimming Pool Finishes

  - Swimming Pool Layout

  - Swimming Pool Piping

  - Swimming Pool Trim

  - Swimming Pool Structural

  - Tower

  - Window and door installation

  - Garage Door installation

  - Structural Carpentry

  - Plaster and Lath

  - Industrial facility

  - Structural Steel

#### * Denotes General, Building, or Residential Certification required in lieu of.

- Electrical (EC)

- Alarm System I (EF)

- Alarm System II (EG)

- Registered Electrical Contractor (ER)

- Registered Electrical Specialty Contractor (Locally Registered) (ET)

- Specialty Electrical (ES)

  - Lighting Maintenance

  - Sign

  - Residential

  - Limited Energy

  - Utility Line

#### Types of Specialty Licenses offered by Manatee County:

- Journeyman Electrician and Plumbing